Ultra Sack –

Self Filling

‘Sand’ Bags

Building Protection

Special features

- Supplied ready to use

- Easy to store

- Require no filling

- Absorb water in 5 – 6 minutes

- Can be re-used

- Indefinite shelf life

Introduction

Ultra Sacks contain a harmless inert gel which expands in contact with water:

- Store flat and dry

- expand when soaked in about 6 mins.

- flatten as they dry out, if hung up.

- packed in 25’s in sealed plastic bags

DESCRIPTION

PRS Ultra Sack is a revolutionary direct replacement for the traditional ‘sand’ bag: it is stored flat ready for instant flood defence – no more sand, filling or weight problems.

The bag is made of high quality woven Jute lined with cotton, which are natural, renewable materials. The bag has a unique, stitched centre to allow the bags to be stacked.

The gel is an inert granulated product commonly used as a soil expander – it has no effect on the environment.

TECHNICAL DATA

Bag Size - dry

62 cm x 35 cm x 6 mm (24” x 13.8”)

Bag Size – wet, maximum

Expands from flat to 100-125 mm (4-5 inches) thick

Packaging (Box of 50 = 330 x 380 x 620mm)

Bag, plastic, sealed in 25 = 150mm thick (6”)

Weight Empty/Full/Re-Dried

0.42 – 0.47 kg/10 - 12 kgs/1.5kgs

Usage Rate – typical examples

25 sacks, = 2 x doorways, 3 across x 4 high – protects against a 250mm flood depth

110 sacks = 10 metre run, 450mm flood depth

DCM – 08/11

Stored Dry & Flat

Lightweight

Property Repair Systems

01626 331351

USES

Flood Control

Remove any debris from the surface. Soak the bags with at least 10 litres of fresh water each to expand and place in layers, like courses of brickwork, making sure that each one overlaps the one beneath by half a bag. Place the bags lengthways and parallel to the water flow at least 25 mm (1 inch) beyond the edge of each opening and butt each firmly to form a dam. Interleave a plastic sheet between layers to improve the seal.

PRECAUTIONS

- minimum wall thickness is one bag length

- bags require at least 10 litres of water each

- over three layers high use the pyramid style, 2 – 3 times as wide as it is high

- place some dry bags inside the building, behind the doors, to absorb any leaks

- To dry, remove from contact with the ground by hanging up over walls or rails.

- Allow 2 – 6 weeks for drying, subject to weather.

Property Repair Systems - 01626 331351

Unit 3, Olympus Business Park, TQ12 2SN